RAYMARINE E-Series Plotter Notes

How do I write/retrieve data to a CompactFlash card?

Your display can save up to 1,200 waypoints, 150 routes and 10 tracks in data base

lists. You may want to save more than this number, or use waypoints, routes or tracks

on another instrument or archive them for safe-keeping. This can be carried out in one

of two ways, either

• By archiving them to a CompactFlash® card to be retrieved again later, or:

• Transferring to another instrument or PC by using NMEA.

When navigation is active, you can also receive waypoints, routes and tracks from

other SeaTalk instruments and from NMEA instruments.

Note: In a networked E-Series system, you archive to and retrieve data from the CompactFlash

card in the display on which you are working.

# Saving information to a card

Important: Raymarine strongly recommend that you save data to a separate CompactFlash card and not to Navionics cards containing cartography.

1. Insert a card into the card slot.

2. Press the DATA button.

3. Press ARCHIVE & TRANSFER.

4. Press SAVE TO CARD.

5. Toggle SELECT LIST to WPT (Waypoint), RTE (Route), or TRK (Track).

6. The database list for your selection appears on the screen.

7. Press the SAVE ALL or SAVE GROUP/ROUTE/TRACK as appropriate. A message

box appears to show that the information is being written to the card.

8. When the transfer is complete a message will appear on the screen.

Note: Pressing the SAVE ALL soft key will save all of a Database list to a card.

Retrieving information from a card

1. Insert a card into the card slot. (See page 2-12 in your manual).

2. Press the DATA button.

3. Press ARCHIVE & TRANSFER.

4. Press RETRIEVE FROM CARD.

5. From SELECT LIST, choose WPT (Waypoint), RTE (Route), or TRK (Track).

6. The card contents for your selection appears on the screen.

7. Use the trackpad (up/down) to select what you want to retrieve.

8. Press RETRIEVE ALL or RETRIEVE GROUP/ROUTE/TRACK as appropriate. A message box appears to show that the information is being written to the display.

9. When the transfer is complete a message will appear on the screen.

Note: Pressing the RETRIEVE ALL soft key, will retrieve all of a Database list to the display.

If whilst retrieving information a selection is already found to exist on the display, a warning message will appear:

The soft keys will change to give you the options of:

• RETRIEVE AS NEW

• REPLACE

• CANCEL RETRIEVE

Select which option you want by pressing the corresponding soft key.

# Erasing information from a card

If you wish to remove information from a card, for example you no longer need it, or the card is full, it can be erased by using the DELETE or DELETE ALL soft keys that can be found on the Retrieve soft key bar. A message will be displayed on screen asking you to confirm that you want to delete the item(s), before it takes place.

How do I show or hide a route or a track?

To show/hide a route or track:

1. Press NAVIGATION OPTIONS, then ROUTES button on the bottom of the screen.

2. Press SHOW/HIDE ROUTES

The Route list is displayed.

3. Select the required route from the list. Its current display status is noted on the right-hand side of the list.

4. Press ROUTE ON CHART to highlight either HIDE or SHOW as appropriate.

Notes: (1) Alternately, move the cursor over the appropriate route and then press HIDE ROUTE.

(2) The active route is always displayed irrespective of the display status.